Department of Electronics and Communication Engineering ECA47 Principles of Digital System Design

# LAB MANUAL

This page is intentionally left blank

INDEX

| Ex.No. | Date | Title | Marks | Staff Sign. |
| --- | --- | --- | --- | --- |
| 1 |  | Verification of Boolean Theorems Using Digital Logic Gates |  |  |
| 2. |  | Verification of De Morgan’s Law Using Basic Logic Gates |  |  |
| 3. |  | # Implementation of Binary To Gray Code Converter |  |  |
| 4. |  | Implementation of Binary Adders and Subtractors |  |  |
| 5. |  | Multiplexer And Demultiplexer |  |  |
| 6. |  | Design of Combinational Logic Circuits |  |  |
| 7. |  | Implementation of Flip Flop Using Logic Gates 7.1 SR Flip Flop 7.2 D Flip Flop 7.3 JK Flip Flop |  |  |
| 8. |  | Implementation of Arithmetic Operations Using 8051 Microcontroller |  |  |
| 9. |  | Interfacing 7 Segment Display With 8051 Microcontroller |  |  |
| 10. |  | Design, Implementation, and Simulation of Combinational Circuits using Verilog HDL |  |  |
| 11. |  | Design, Implementation, and Simulation of Shift Register Using Verilog HDL |  |  |
| 12. |  | Design, Implementation, and Simulation of Mod 5 Counter Using Verilog HDL |  |  |

Ex.No.-1  VERIFICATION OF BOOLEAN THEOREMS USING DIGITAL LOGIC GATES

Date:

AIM:

To Implement and verify the following Boolean theorems using logic gates and the Digital Trainer Kit.

- Implement the commutative theorem (A + B = B + A) using logic gates.

- Implement the Absorption Law  A+A’B=A+B

- Implement the associative theorem (A + B + C) = (A + B) + C = A + (B + C)) using logic gates

# APPARATUS REQUIRED:

| SL. NO. | COMPONENT | SPECIFICATION | QTY. |
| --- | --- | --- | --- |
| 1. | AND GATE | IC 7408 | 1 |
| 2. | OR GATE | IC 7432 | 1 |
| 3. | NOT GATE | IC 7404 | 1 |
| 4. | IC TRAINER KIT | - | 1 |
| 5. | CONNECTING WIRES | - | As required |

# THEORY:

BASIC BOOLEAN LAWS

## Commutative Law

The binary operator OR  is said to be commutative if,

A+B = B+A

## Absorption Law

A+A’B=A+B

## Associative Law

The binary operator OR is said to be associative if,

A+(B+C) = (A+B)+C

COMMUTATIVE

# TRUTH TABLE

| I/P |  | LHS (O/P) | RHS (O/P) |
| --- | --- | --- | --- |
| A | B | A+B | B+A |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 |

# LOGIC DIAGRAM

ABSORPTION

# TRUTH TABLE

| I/P |  |  |  | LHS (O/P) | RHS(O/P) |
| --- | --- | --- | --- | --- | --- |
| A | B | A' | A'B | A + (A'B) | A + B |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 |

# LOGIC DIAGRAM

# TRUTH TABLE

| I/P |  |  |  | LHS (O/P) |  | RHS (O/P) |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | C | B+C | A + (B+C) | A + B | (A + B ) + C |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |

# LOGIC DIAGRAM

# PROCEDURE:

- Obtain the required IC along with the Digital trainer kit.

- Connect zero volts to GND pin and +5 volts to Vcc .

- Apply the inputs to the respective input pins.

- Verify the output with the truth table.

# RESULT:

Thus the above stated Boolean laws are verified.

Ex.No.-2 VERIFICATION OF DE MORGAN’S LAW USING BASIC LOGIC GATES

Date:

AIM:

To verify De Morgan's law using basic logic gates and construct the associated truth tables.

# APPARATUS REQUIRED

| SL. NO. | COMPONENT | SPECIFICATION | QTY. |
| --- | --- | --- | --- |
| 1. | AND GATE | IC 7408 | 1 |
| 2. | OR GATE | IC 7432 | 1 |
| 3. | NOT GATE | IC 7404 | 1 |
| 4. | IC TRAINER KIT | - | 1 |
| 5. | CONNECTING WIRES | - | As required |

# THEORY

## De-Morgan’s theorem:

- The complement of the sum is equal to the product of the individual complements.

(A+B)’=A’.B’

- The complement of the product is equal to the sum of the individual complements.

(A.B)’=A’+B’

# TRUTH TABLE

| I/P |  |  | LHS (O/P) |  |  | RHS (O/P) |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | (A + B) | (A + B)' | A' | B' | A'.B' |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 |

# LOGIC DIAGRAM

# TRUTH TABLE

| I/P |  |  | LHS (O/P) |  |  | RHS (O/P) |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | (AB) | (AB)' | A' | B' | A' + B' |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 |

# LOGIC DIAGRAM

# PROCEDURE:

  - Obtain the required IC along with the Digital trainer kit.

  - Connect zero volts to GND pin and +5 volts to Vcc .

  - Apply the inputs to the respective input pins.

  - Verify the output with the truth table.

# RESULT:

Thus the above stated Demorgan’s Theorem is verified .

# Ex.No.-3	IMPLEMENTATION OF BINARY TO GRAY CODE CONVERTER

# Date:

# AIM:

To design and implement a 4-bit Binary to Gray Code Converter using ICs on a Digital trainer kit and verify its operation experimentally.

# APPARATUS REQUIRED:

| SL.NO. | COMPONENT | SPECIFICATION | QTY |
| --- | --- | --- | --- |
| 1. | X-OR GATE | IC 7486 | 1 |
| 2. | NOT GATE | IC 7404 | 1 |
| 3. | IC TRAINER KIT | - | 1 |
| 4. | CONNECTING WIRES | - | 35 |
| 5. | BREAD BOARD |  | 1 |

### PROCEDURE:

- Obtain the required IC and Digital Trainer Kit.

- Connect +5 V to VCC and 0 V to GND.

- Make the circuit connections as per the circuit diagram.

- Apply the binary inputs to the input pins.

- Observe the Gray code outputs.

- Verify the outputs using the truth table.

# TRUTH TABLE:

| Binary Input |  |  |  | Gray Code Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3 | B2 | B1 | B0 | G3 | G2 | G1 | G0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

## K-Map for G3                                                                   K-Map for G2

G3 = B3

K-Map for G1     							 K-Map for G0

LOGIC DIAGRAM:

# RESULT:

Thus the above stated Binary to Gray code Converter is implemented.

Ex.No.-4  IMPLEMENTATION OF BINARY ADDERS AND SUBTRACTORS

Date:

AIM

To design a half adder and a full subtractor circuit on the Digital Trainer Kit.

# APPARATUS REQUIRED:

| Sl.No. | COMPONENT | SPECIFICATION | QTY. |
| --- | --- | --- | --- |
| 1. | AND GATE | IC 7408 | 1 |
| 2. | X-OR GATE | IC 7486 | 1 |
| 3. | NOT GATE | IC 7404 | 1 |
| 4. | OR GATE | IC 7432 | 1 |
| 5. | IC TRAINER KIT | - | 1 |
| 6. | CONNECTING WIRES | - | As required |

# HALF ADDER:

# TRUTH TABLE:

| A | B | SUM | CARRY |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Sum = A’B+ AB’= A⊕B

Carry = A . B

# PROCEDURE:

- Connections are given as per the logic diagram

- Input are given to the circuit making high ‘1’ i.e. +5 or Vcc Supply to the 14th pin and for low ‘0’ i.e. GND to the 7th pin of the Gate IC

- Depending upon the truth table, if the LED Glow it represents 1 and else it represents ‘0’

- Verify the truth table as given

# LOGIC DIAGRAM FOR HALF ADDER:

# FULL SUBRACTOR: TRUTH TABLE:

| A | B | C | DIFFERENCE | BORROW |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Difference	= A’B’C +A’BC’ + AB’C’ + ABC

=A’(B’C+BC’)+A(B’C’+BC)

=A’(B⊕C)+ A (B⊕C)’

=A⊕B⊕C

Borrow = A’B’C +A’BC’ + A’BC + ABC

= C(A’B’+ AB)+ A’B (C+C’)

= C(A⊕B)’+ A’B

# LOGIC DIAGRAM FOR FULL SUBTRACTOR:

# RESULT:

Thus, the half adder and full subtractor were designed and verified successfully using Truth table.

Ex.No 5. MULTIPLEXER AND DEMULTIPLEXER

#### Date:

#### AIM:

To implement the 2:1 multiplexer and  1:2 Demultiplexer circuit on the Digital Trainer Kit.

# APPARATUS REQUIRED:

| SL.NO. | COMPONENT | SPECIFICATION | QTY. |
| --- | --- | --- | --- |
| 1. | AND GATE | IC 7408 | 1 |
| 2. | OR GATE | IC 7432 | 1 |
| 3. | NOT GATE | IC 7404 | 1 |
| 2. | IC TRAINER KIT | - | 1 |
| 3. | PATCH CORDS | - | 32 |

# PROCEDURE:

  - Connections are given as per circuit diagram.

  - Logical inputs are given as per circuit diagram.

  - Observe the output and verify the truth table.

# LOGIC DIAGRAM OF 2:1 MULTIPLEXER

# TRUTH TABLE:

| Select Input S | Y = OUTPUT |
| --- | --- |
| 0 | D0 |
| 1 | D1 |

# LOGIC DIAGRAM OF 1:2 DEMULTIPLEXER

# TRUTH TABLE:

| Input S | Outputs |  |
| --- | --- | --- |
|  | Y0 | Y1 |
| 0 | D | 0 |
| 1 | 0 | D |

# RESULT:

Thus the multiplexer and demultiplexer using logic gates are designed and implemented